Plasma-enhanced atomic layer deposition system

(PLASMA-ENHANCED ATOMIC LAYER DEPOSITION)

System Introduction

The plasma-enhanced atomic layer deposition system deposit films with high dielectric constant. By changing the precursor, it is possible to deposit different films. Currently, this system is available for HfO2 and Al2O3 film deposition, with a thickness between 20~100A.

- Full name: Plasma-enhanced atomic layer deposition system

- Location: class 10000

- Engineer: Mr. Jin-Long Hsieh

- ICE number: (O) 03-5773693 ext. 7520

- Substitute engineer: Mr. Wen-Cheng Chiu

- ICE number: (O) 03-5773693 ext. 7569

- Chemicals used on machine:

- Bulk gases:  N2, Ar, O2,

- Specialty gases:

- Bottled chemicals: 	Metal-organic compounds,

- Process mode or reaction chamber: Plasma mode

- Reaction chamber vacuum range: 5e-2 torr

- Vacuum pump: Oil pump

- Substances in exhaust emissions: C, N, O

- Separate local scrubber:  Provided by facility

- Reaction chamber temperature range:  250~400°C

- Type of cooling system: 	Cooling water supplied by facility

- Other special hazards: 	Organic compounds, gases, etc.

- Manufacturer/agent: 	Syskey Technology

- Agent engineer: 	Pei-Hsiang Yeh, Tel: 03-5590169

- Update date: 2/7/2024