8” back-end chemical wet etching workstation CF-C09

Knowledge Evaluation Form

- Wearing of Protective Gear

  - Is protective gear being worn properly?

  - Is protective gear cleaned, returned to designated locations, and placed in order according to regulations after use?

- Awareness of Equipment Hazards and Crisis Management

  - Temperature control for discharge of sulfuric acid tank solution

  - Notes on B.O.E tank discharge

  - Purpose of the Material Safety Data Sheet (MSDS)?

  - Types of chemicals that can be used in each tank (explain in detail the part about sulfuric tank) and hazard awareness?

  - Location and correct use of the body shower and eye washer of the back-end wet bench process area

  - Method of use of hexafluorine and diphoterine?

  - Method of use and usage timing of calcium gluconate?

  - Methods to stop the machine in emergency situations

- Relevant Regulations on Pollution Control at the Equipment-end and Pollution Control Principles at Process-end

  - Location of the dedicated shelf and cassette in the back-end wet bench process area? Restrictions on the spin dryer?

  - Can breakage be handled in the wet bench process tank? If yes, what is its related drying facility?

  - Restrictions on chemical use at TSRI? (application to bring own or to temporarily store)

  - Can wafers (including breakage) that have been processed and handled at the back-end process area be processed at the front-end process bench?

- Basic operational knowledge of each chemical tank

  - Which metal films can be handled at the H2SO4 tank in the back-end process area?

  - Which metal films can be handled at the B.O.E. tank in the back-end process area?

  - How to replace each type of solution that is permitted to be soaked in the back-end process area? (randomly select 1 recipe to demonstrate)

  - What is the use of vacuum pens in the Wet Bench area?

- Operational procedures for chemical replacement and preparation of various common metal film removal or etching processes

- Operational procedures for QDR tank

  - In the wet cleaning or etching process, the QDR method is normally used for cleaning wafers. What is QDR?

  - What is the purpose of QDR in the wet cleaning or etching process?

  - Carry out a wafer cleaning process (hands-on demonstration)

- Operational procedures for Spin dryer

  - How to carry out the wafer drying process in the back-end process area?

  - Explain the action principles of drying the wafer with the spin dryer and explain the purpose of the static elimination installation

  - Explain the usage timing of Rinse Mode and Dry Mode

  - Operational procedures for Spin dryer (hands-on demonstration)

- Chemical supply system and use and handling of small-vial chemicals

  - Chemical ratios of each recipe and usage timing of H2SO4tank and B.O.E. tank

  - For the Wet Bench in the back-end process area, which types of chemicals can only be supplied by the central supply system?

- Inspection of equipment before and after use

Inspection of machine before use

  - Identification of machine light signal display (green light: functioning normally; red light: under repair)

  - Check the use log sheet (able to know the status of each chemical tank)

Inspection of equipment after use

- Record into log sheet after use

- Which type of form to fill out when there are issues with the equipment?