| Document Title: | Machine SOP (CF-E10 8-inch front-end polysilicon/dielectric dry etching machine) |
| --- | --- |
| Document No.: | Q3-NL04 |
| Draft Dept.: | Film etching division |
| Draft Date: | 2019-02-15 |

Document Revision History

| Version | Edited by | Effective date | Approve document No. | Revision/change description | Revised Page |
| --- | --- | --- | --- | --- | --- |
| 1.0 | Fu-Guo Xue | 2019-02-20 | IS108006 | New release | --- |
| 1.1 | Fu-Guo Xue | 2019-06-05 | IS108024 | Replaced “Instrument” in the title in header with “Machine SOP” | P1-7 |
| 1.2 | Fu-Guo Xue | 2023-05-11 | IS112011 | - Updated the description in Article 3 under III. Rights and Responsibilities | P1 |

- Purpose:

Define 8” front-end polysilicon/dielectric dry etching machine practices to ensure production quality.

- Scope:

Applicable to 8” front-end polysilicon/dielectric etching machine.

- Rights and Responsibilities:

- Organization responsibilities: The process engineer is responsible for the drafting and revision of specifications.

- Equipment manager is responsible for handling anomalies to maintain normal operation.

- Qualified operator: Operators passing 8” front-end polysilicon/dielectric etching machine operation test.

- Relevant Documents:

Lam2300 - 8” front-end polysilicon/dielectric etching machine Operation manual.

- Introduction of Machine Parts:

Gas panel system

Polysilicon etching chamber

Dielectric etching chamber

Cassette load port module

Vacuum transfer module

Vacuum/atmosphere exchange module

Atmosphere transfer module

Gas panel system

Polysilicon etching chamber

Dielectric etching chamber

Cassette load port module

Vacuum transfer module

Vacuum/atmosphere exchange module

Atmosphere transfer module

Ashing chamber

Ashing chamber

- Operation Content:

- Inspect machine condition and current status (green card: in use; red card: under maintenance; yellow card: under test). Only qualified operators are allowed to swipe the card to boot.

- Lay the wafer wafer in the cassette and place in Port 1 or 2 (as shown in Fig. 1). Make sure the cassette and wafer are loaded correctly.

Port2

Port2

Port1

Port1

Fig. 1: Cassette load port module

Fig. 1: Cassette load port module

- Select process parameters from Operation page (as shown in Fig. 2): (1) Enter Carrier ID and Lot ID (2) Select required parameters from Wafer flow (3) Check wafer quantity (4) Start operation.

Fig. 2: Operate mode page

Fig. 2: Operate mode page

(4)

(4)

(1)

(2)

(3)

(1)

(2)

(3)

- Check wafer status in Operation page: 〔Gray: process not executed; green: process completed; blue: wafer in transfer; yellow: anomaly occurred in process〕(as shown in Fig. 3).

Fig. 3: Wafer status

Fig. 3: Wafer status

- If any error occurs during the experiment, press “Abort” or “Stop” (as shown in Fig. 4) and contact machine engineer.

Fig. 4: Abort and stop page

Fig. 4: Abort and stop page

- After the process is completed (when wafer field turns green), open Port 1 or 2 to take out the wafer.

- Alarm troubleshooting process: Check and record the message displayed on Alarm page (as shown in Fig. 5) and notify the engineer.

Fig. 5: Alarm page

Fig. 5: Alarm page

- MES swipe to turn off the machine

- Notes:

- Please notify the person in charge when an anomaly cannot be determined and solved.

- Make sure to refer to machine SOP and follow relevant regulations when using the machine.

- Reference forms and attachments:

- Q4-NL02 Machine management card

- Q4-NL03 Machine evaluation checklist

- Q4-NL04 Machine checklist

- Q4-NL06 Anomaly and Preventive/ correction action list