Notes on TOPCON Surfscan

- To operate the machine by yourself, you must be trained in the use of the equipment and pass a test.

- This equipment can only provide surface contamination analysis for 6- and 8-inch wafers.

- This equipment is unable to provide surface contamination analysis for patterned or metal film materials.

- When using the machine, you must use a vacuum pen to select chips. Do not use chip clamps to pick up wafers to avoid affecting data determination.

- Use the standard cassette of the machine to prevent the equipment from crashing.

- Do not remove the cassette during the operation of the equipment.

- Do not remove the cassette to pick up the chips until the end-of-operation alarm goes off.

- If possible, do not use printers to save ink and dust-free paper.