| Document Title: | System SOP (CF-T26 Backend CVD system) |
| --- | --- |
| Document No.: | Q3-NL04 |
| Draft Dept.: | Film etching division |
| Draft Date: | 2019-02-15 |

Document Revision History

| Version | Edited by | Effective date | Approve document No. | Revision/change description | Revised Page |
| --- | --- | --- | --- | --- | --- |
| 1.0 | Po-Hao Chu | 2019-02-20 | IS108006 | New release | --- |

- Purpose:

Define the operating specifications of the Backend CVD system to ensure high operation quality.

- Scope:

Applicable to Backend CVD system.

- Rights and Responsibilities:

- Organization responsibilities: The process engineer is responsible for the drafting and revision of specifications.

- Equipment manager is responsible for handling anomalies to maintain normal operation.

- Qualified operator: Operators passing the Backend CVD system evaluation system test.

- Relevant Documents:

Backend CVD system Operation manual.

- SOP:

- How to edit a Recipe

- Access the Homepage and click “Applied Material Centura System 5200” on top. Enter the password first.

- USER Name: AMAT

- Password: 5989

- Access the Home screen and select Program\Process Program.

- You can see a menu displaying Recipe name on the screen.

- Select a recipe to edit steps.

For example, when selecting “PE-XXX-XK” in the recipe, the user can edit and operate all green text in each step.

- Click “Header/Exchange” on the screen to edit the operating environment of this recipe. Remember to change “FROZEN” to “Modify” before editing.

- How to edit Sequence

- Click “Program\Wafer Sequencing” on the screen

- You can see a menu displaying Sequencing name on the screen.

- Click any Program Name to edit the path to run the wafer. For example, when selecting “PE-XXX-XK-X”:

- Step 1 means Loadlock A or B enters the cassette

- Step 2 means the wafer enters Chamber C to run Recipe “PE-XXX-XK-X”

- Step 3 means after the process ends, the wafer exits the Chamber and then runs a clean recipe

- Step 4 means the wafer enters Chamber E to run “cool 120 SEC” for cooldown

- Step 5 means the wafer returns to the original cassette

- How to edit Lot name

- Click “Program\Lot Sequence Control” on the screen

- You can see a menu displaying Lot name on the screen.

- Enter “PE-XXX-XK-X” in Add new lot name and the same text will appear on the left and a corresponding field on the right. The user then can select “PE-XXX-XK-X” Sequencing edited before.

- Click “Program\Enter Lot Name For” on the screen to access the required Recipe.

- Click any cassette; you can click Recipe according to the actual wafer position.

- How to load/unload

- Loading a wafer in a Cassette (A or B): when loading a wafer on Load-lock, place the Cassette in the groove and gently push outward and backward to make sure it is placed correctly.

P1: When placing on Loadlock B (gently push outward and backward); place a wafer in the opposite direction when using Loadlock A.

- Select Wafer sequence to correspond to Lot name

P2: Set Lot name and the corresponding Wafer sequence in the indicated field.

- And set Wafer Lot name to run on “Enter Lot Names for Presented Wafers” screen.

- Click “Load/Unload A or B”

- Then click “Run (Go) Button” to start the process.

- Note:

- Please notify the person in charge of the system when an anomaly cannot be determined and solved.

- Make sure to refer to machine SOP and follow relevant regulations when using the machine.

- Reference forms and attachments:

- Q4-NL02 Machine management card

- Q4-NL03 Machine evaluation checklist

- Q4-NL04 Machine checklist

- Q4-NL06 Anomaly and Preventive/ correction action list