PCB Fabrication Review Form

Objective

- This form is used to remind the designers some notices about PCB design, layout, DRC and fabrication.

- We hope the user can raise the PCB design successful rate and obtain complete learning result.

- Please self-check the list on this form and fill in the appropriate cell carefully.

Software information

PADS:

- Encourage users to use TSRI provided PADS software. About PADS, TSRI provides template files, software consulting, e-Learning, and winter / summer vacation training courses.

Altium:

- Please use versions after 2012 (including).

- The design of version before 2012 might not be compatible with TSRI’s template, and generates too many DRC errors.

- After 2015/03, there is no update for the Altium template, please use PADS software instead.

- Please set up the Design Rules by oneself and make sure to comply with the DRC listed in TSRI’s web “PCB Fabrication - Design Rules Manual”.

Allegro:

- No template.

- If the user wants to use the software (Such as Allegro or Eagles, etc.), please set up the Design Rules by oneself and make sure to comply with the DRC listed in TSRI’s web Design Rule manual.

Other Software:

- Not Acceptable.

[1] Name and Phone no. of Applicant

| PCB Identification Code# | 4L_115E_A0001 |
| --- | --- |
| Applicant Name | Jack Smith |
| Applicant Phone No. | (0912)345678 |
| PCB Layout software name /version | PADS VX.2.16 |

# PCB Identification Code Naming rule:

Process Code_year batch_serial number on”PCB Fabrication Application Form”.

Ex: 4L_115E_A0001

Mark this code at “Silkscreen top” upper left corner with a frame to enclose it.

[2] Check the following items

Self-check items, no matter there is the item in your design or not.

If you checked, click the box, .

Red Characters show error-prone items.

| Item | Check | Comments |
| --- | --- | --- |
| PCB Identification Code | ☐ | Exists, and the naming rule is correct. Rule: Process Code_year batch_serial number on”PCB Fabrication Application Form”. Ex: 4L_115E_A0001 |
|  | ☐ | This code should be the same in: “PCB Fabrication Application From” (web), “Silkscreen top”, “PCB Design e-file”, and “PCB Fabrication Review Form”. No drill is allowed on “PCB Identification Code” |
| BO Frame ME Frame | ☐ | BO frame and ME frame are existing. |
|  | ☐ | BO frame, only exists on BO layer, no BO frame on the other layer. ME frame, only exists on ME layer, no ME frame on the other layer. |
|  | ☐ | Line width = 0.1 mm |
|  | ☐ | The distance of BO frame and ME frame = 1.5 mm (center of line to center of line |
| ME Frame Size | ☐ | Should be the same as that of web the cell of “PCB Area Units” on “PCB Application Form”. |
| PCB Cut Out | ☐ | Gerber file of BO layer shows “cut out” characters. |
| Drill Table | ☐ | Existing, does not overlap with any figures of other layers , and must be outside ME frame. |
|  | ☐ | 0.2mm ≤ Size of holes ≤ 6mm |
|  | ☐ | Position Holes：NPTH (Non-Plated through Hole) No. =4，1mm ≤ Size of Position Holes ≤ 6mm。 |
| Solder Mask | ☐ | There is solder mask opening on COB*. |
|  | ☐ | There is solder mask opening on PAD, which is for soldering use. |
|  | ☐ | If there is no solder material on whole layer, be sure to draw solder mask. (the solder mask is negative) |
| Net name | ☐ | Every object on metal layers iss assigned to a net. (Including traces, copper planes, and any figures.) |
| PAD Via | ☐ | PAD with via is not encouraged, unless it is necessary for design. |
| Silkscreen Bottom | ☐ | SSB# file size must >= 0.5KB.. If there is no data, please add “designed by user name or lab name. Ex: design by ABC Lab。 |
| Layer without any data | ☐ | ☐ There are data of all Layer. ☐ Layer has no data. Add Board Outline to this layer, and Upload the Gerber file without any data. The Gerber file of this layer should not have the same name as other file. |
| New Layer | ☐ | ☐ no new layer ☐ Layer is the new layer. Ex: Altium slot layer |

# SST, SSB: Silkscreen Top, Silkscreen Bottom

* COB: Chip On Board

[3] Self Examination

Circle the design rules verified. No matter there is the item in your design or not.

If you checked, click the box, .

Red Characters show error-prone items.

Afterwards, upload DRC Report file。

M: Must  O:Optional

| Item | Verified | Rule Name | Code | Remarks | Calculation Sign | Size (mm) | Properties M / O |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Metal Line | ☐ | MT.W.1 | MW | Metal Line Width | ≧ | 0.10 | M |
|  | ☐ | MT.S.1 | MS | Metal Line to Line distance | ≧ | 0.10 | M |
|  | ☐ | MT.BO.E.1 | MB | Metal Line to Board Outline distance | ≧ | 0.30 | M |
| Copper | ☐ | MT.S.2 | MC | Copper to Metal distance (FR4) | ≧ | 0.18 | M |
|  |  |  |  | Copper to Metal distance (RO) | ≧ | 0.17 | M |
|  | ☐ | MT.BO.E.1 | CB | Copper to Board Outline distance | ≧ | 0.30 | M |
| Plated Through Hole (PTH) and Ring | ☐ | DR.W.1 | D1 | PTH minimum size | ≧ | 0.20 | M |
|  | ☐ | DR.W.2 | D1 | PTH maximum size | ≦ | 6.00 | M |
|  | ☐ | DR.DR.S.1 | D4 | PTH to PTH distance | ≧ | 0.30 | M |
|  | ☐ | DR.MT.S.1 | D5 | PTH to Metal Line distance | ≧ | 0.30 | M |
|  | ☐ | DR.BO.E.1 | D6 | PTH to Board Outline distance | ≧ | 0.50 | M |

(continued, next page)

| Item | Verified | Rule Name | Code | Remarks | Calculation Sign | Size (mm) | Properties M / O |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Plated Through Hole (PTH) and Ring | ☐ | MT.W.2 | D2 | PTH external Metal layer hole ring Line width | ≧ | 0.10 | M |
|  | ☐ | MT.SO.E.1 | D3 | PTH ring to Solder Mask distance | ≧ | 0.10 | O |
| Non-Plated Through Hole (NPTH) | ☐ | DR.W.3 | N1 | NPTH minimum size | ≧ | 0.50 | M |
|  | ☐ | DR.W.4 | N1 | NPTH maximum size | ≦ | 6.00 | M |
|  | ☐ | DR.DR.S.2 | N2 | NPTH to NPTH distance | ≧ | 0.30 | M |
|  | ☐ | DR.RU.S.2 | N3 | NPTH to Board Outline distance | ≧ | 0.50 | M |
|  | ☐ | DR.SO.E.2 | N4 | NPTH to Solder Mask distance | ≧ | 0.12 | O |
| Slot (PTH / NPTH) | ☐ | DR.W.5 | O1 | Slot Drill Hole minimum size | ≧ | 0.55 | M |
|  | ☐ | DR.W.6 | O1 | Slot Drill Hole maximum size | ≦ | 6.00 | M |
|  | ☐ | DR.DR.S.3 | O2 | Slot Drill Holes center to center distance | ≧ | > O1 | M |
|  | ☐ | DR.DR.S.4 | O3 | Slot to Slot distance | ≧ | 0.50 | M |
|  | ☐ | DR.RU.S.3 | O4 | Slot to Board Outline distance | ≧ | 0.80 | M |
| Solder Mask | ☐ | MT.SO.E.1 | S1 =D3 | Metal PAD Solder Mask distance | ≧ | 0.10 | O |
|  | ☐ | SO.W.1 | S2 | Solder Mask Line width | ≧ | 0.10 | O |

(continued, next page)

| Item | Verified | Rule Name | Code | Remarks | Calculation Sign | Size (mm) | Properties M / O |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Silk screen | ☐ | SK.W.1 | SkW | Text Line width | ≧ | 0.15 | O |
|  | ☐ | SK.H.1 | SkH | Text Height | ≧ | 1.00 | O |
|  | ☐ | SK.MT. S.1 | SkM | Text to PAD的distance | ≧ | 0.20 | O |
| BO Layer | ☐ | BO.W.1 | BOW | Board Outline Line width | = | 0.10 | M |
|  | ☐ | BO.H.1 | NH | Position Hole diameter | ≧ | 1.00 | M |
|  | ☐ | BO.H.2 | NH | Position Hole property | = | NPTH | M |
|  | ☐ | CO.R.1 | COR | PCB cut out corner arc diameter | ≧ | 1.00 | M |
|  | ☐ | CO.W.1 | COW | PCB Cut Out width | ≧ | 0.80 | M |
|  | ☐ | CO.L.1 | COL | PCB Cut Out Length | ≧ | 0.80 | M |
|  | ☐ | CO.M.E.1 | COM | PCB Cut Out to Metal distance | ≧ | 0.30 | M |
|  | ☐ | CO.B.E.1 | COM | PCB Cut Out to Board Outline distance | ≧ | 0.30 | M |
|  | ☐ | CO.T.1 | COT | Text in Cut Out | - | - | M |
| ME Layer | ☐ | ME.W.1 | MEW | ME Frame Line width | = | 0.10 | M |
|  | ☐ | ME.S.1 | MES | ME Frame to Board Outline distance | = | 1.50 | M |

(end)